Lab 1: Logic Gates

ITI 1100 C - Digital Systems

Fall 2016
School of Electrical Engineering and Computer Science University of Ottawa

Course Coordinator: Dr. Ahmed Karmouch

Teaching Assistants:

TA 1 Name
TA 2 Name

Group 6

Felix Singerman 7970742

Abigail Woods

Experiment Date: February 2, 2016

Submission Date: February 23, 2016

Objectives

- Construct simple combinational logic circuits from a schematic.

- Experimentally determine the functional operation of simple combinational logic circuits.

- Identify equivalent logic gates to those produced by various circuit configurations from the resulting truth table.

- Connect various gates together to create simple logic functions.

- Analyze combinational logic circuits and predict their operation.

- Construct and test more complex combinational logic circuits.

Equipment & Components

- Quartus II 13.0 Service-Pack 1

- Altera DE2-115 card

Circuit Diagrams

Part I – Combinational Logic Circuit Construction

One-Chip Logic Circuit

- Figure 1: Screen-shot of One-chip logic circuit diagram  (Figure 5.1.1)

Two-Chip Logic Circuit

- Figure 2: Screen-shot of Two-chip logic circuit diagram  (Figure 5.1.2)

Three-Chip Logic Circuit

Figure 3: Screen-shot of Three-chip logic circuit diagram  (Figure 5.1.3)

- Part II - Combinational Logic Circuits Analysis

Exclusive OR Circuit

Figure 4: Screen-shot of Exclusive OR logic circuit diagram  (Figure 5.1.5 of Lab Manual)

AND Circuit

Figure 5: Screen-shot of AND logic circuit diagram  (Figure 5.1.6 of Lab Manual)

OR Circuit

Figure 6: Screen-shot of OR logic circuit diagram  (Figure 5.1.7 of Lab Manual)

Multiple Output Circuit

Figure 7: Screen-shot of Multiple Output logic circuit diagram  (Figure 5.1.8 of Lab Manual)

Experimental Data and Data Processing

Part I – Combinational Logic Circuit Construction

One-Chip Logic Circuit

- Figure 8: Simulation output waveform of one-chip circuit

Table 1: Experimental data observed from MAX 7000 Circuit Board

| Input Given From Dip Switches |  |  | Observed Output from LED’s |
| --- | --- | --- | --- |
| A | B | C | Output |
| 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 |
| 0 | 1 | 0 | 1 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 0 | 1 |
| 1 | 0 | 1 | 1 |
| 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |

Two-Chip Logic Circuit

- Figure 9: Simulation output waveform of two-chip circuit

- Table 2: Experimental data observed from MAX 7000 Circuit Board

| Input Given From Dip Switches |  |  |  | Observed Output from LED’s |
| --- | --- | --- | --- | --- |
| A | B | C | D | Output |
| 0 | 0 | 0 | 0 | 1 |
| 0 | 0 | 0 | 1 | 1 |
| 0 | 0 | 1 | 0 | 1 |
| 0 | 0 | 1 | 1 | 1 |
| 0 | 1 | 0 | 0 | 1 |
| 0 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 0 | 1 |
| 0 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 1 |
| 1 | 0 | 0 | 1 | 1 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 0 | 1 | 1 | 1 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 0 | 1 | 1 |
| 1 | 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 | 0 |

Three-Chip Logic Circuit

- Figure 10: Simulation output waveform of three-chip circuit

- Table 3: Experimental data observed from MAX 7000 Circuit Board

| Input Given From Dip Switches |  | Observed Output from LED’s |
| --- | --- | --- |
| A | B | Output |
| 0 | 0 | 1 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

- Part II - Combinational Logic Circuits Analysis

Exclusive OR Circuit

- Figure 11: Simulation output waveform of exclusive OR circuit

- Table 4: Experimental data observed from MAX 7000 Circuit Board

| Input Given From Dip Switches |  | Observed Output from LED’s |
| --- | --- | --- |
| A | B | L |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |

AND Circuit

- Figure 12: Simulation output waveform of AND circuit

- Table 5: Experimental data observed from MAX 7000 Circuit Board

| Input Given From Dip Switches |  |  |  | Observed Output from LED’s |
| --- | --- | --- | --- | --- |
| A2 | B2 | C2 | D2 | V2 |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 1 | 0 |
| 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 |
| 0 | 1 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 |
| 1 | 0 | 1 | 1 | 1 |
| 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 |

OR Circuit

- Figure 13: Simulation output waveform of OR circuit

- Table 6: Experimental data observed from MAX 7000 Circuit Board

| Input Given From Dip Switches |  |  | Observed Output from LED’s |
| --- | --- | --- | --- |
| A | B | C | P1 |
| 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 |
| 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 0 | 1 |
| 1 | 0 | 1 | 1 |
| 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |

Multiple Output Circuit

- Figure 14: Simulation output waveform of Multiple Output Circuit

- Table 7: Experimental data observed from MAX 7000 Circuit Board

| Input Given From Dip Switches |  |  | Observed Output from LED’s |  |
| --- | --- | --- | --- | --- |
| A3 | B3 | D3 | S3 | C3 |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |

Comparison of Theoretical Data and Experimental Data

Part I – Combinational Logic Circuit Construction

One-Chip Logic Circuit

- Table 8: Comparison of Theoretical and Experimental results for one-chip logic circuit

| Inputs |  |  | Expected Results | Actual Results |
| --- | --- | --- | --- | --- |
| A | B | C | Output | Output |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 |
| 0 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 1 | 1 |
| 1 | 0 | 1 | 1 | 1 |
| 1 | 1 | 0 | 1 | 1 |
| 1 | 1 | 1 | 1 | 1 |

The results observed experimentally for one chip circuit from MAX 7000 FGPA were identical to results obtained theoretically as expected.

Two-Chip Logic Circuit

- Table 9: Comparison of Theoretical and Experimental results for two-chip logic circuit

| Inputs |  |  |  | Expected Results | Actual Results |
| --- | --- | --- | --- | --- | --- |
| A | B | C | D | Output | Output |
| 0 | 0 | 0 | 0 | 1 | 1 |
| 0 | 0 | 0 | 1 | 1 | 1 |
| 0 | 0 | 1 | 0 | 1 | 1 |
| 0 | 0 | 1 | 1 | 1 | 1 |
| 0 | 1 | 0 | 0 | 1 | 1 |
| 0 | 1 | 0 | 1 | 1 | 1 |
| 0 | 1 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 1 | 1 |
| 1 | 0 | 0 | 1 | 1 | 1 |
| 1 | 0 | 1 | 0 | 1 | 1 |
| 1 | 0 | 1 | 1 | 1 | 1 |
| 1 | 1 | 0 | 0 | 1 | 1 |
| 1 | 1 | 0 | 1 | 1 | 1 |
| 1 | 1 | 1 | 0 | 1 | 1 |
| 1 | 1 | 1 | 1 | 0 | 0 |

The results observed experimentally for one chip circuit from MAX 7000 FGPA were identical to results obtained theoretically as expected.

Three-Chip Logic Circuit

- Table 10: Comparison of Theoretical and Experimental results for three-chip logic circuit

| Inputs |  | Expected Results |  | Actual Results |
| --- | --- | --- | --- | --- |
| A | B | OUTPUT | OUTPUT |  |
| 0 | 0 | 1 | 1 |  |
| 0 | 1 | 0 | 0 |  |
| 1 | 0 | 0 | 0 |  |
| 1 | 1 | 1 | 1 |  |

The results observed experimentally for one chip circuit from MAX 7000 FGPA were identical to results obtained theoretically as expected.

- Part II - Combinational Logic Circuits Analysis

Exclusive OR Logic Circuit

- Table 11: Comparison of Theoretical and Experimental results for exclusive OR logic circuit

| Inputs |  | Expected Results |  | Actual Results |
| --- | --- | --- | --- | --- |
| A | B | L | L |  |
| 0 | 0 | 0 | 0 |  |
| 0 | 1 | 1 | 1 |  |
| 1 | 0 | 1 | 1 |  |
| 1 | 1 | 0 | 0 |  |

The results observed experimentally for one chip circuit from MAX 7000 FGPA were identical to results obtained theoretically as expected.

AND Logic Circuit

- Table 12: Comparison of Theoretical and Experimental results for AND logic circuit

| Inputs |  |  |  | Expected Results | Actual Results |
| --- | --- | --- | --- | --- | --- |
| A2 | B2 | C2 | D2 | V2 | V2 |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 | 1 |
| 0 | 1 | 0 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 0 | 0 | 0 |
| 0 | 1 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 | 0 | 0 |
| 1 | 0 | 1 | 0 | 0 | 0 |
| 1 | 0 | 1 | 1 | 1 | 1 |
| 1 | 1 | 0 | 0 | 0 | 0 |
| 1 | 1 | 0 | 1 | 0 | 0 |
| 1 | 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |

The results observed experimentally for one chip circuit from MAX 7000 FGPA were identical to results obtained theoretically as expected.

OR Logic Circuit

- Table 13: Comparison of Theoretical and Experimental results for OR logic circuit

| Inputs |  |  | Expected Results | Actual Results |
| --- | --- | --- | --- | --- |
| A | B | C | P1 | P1 |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 |
| 0 | 1 | 0 | 0 | 0 |
| 0 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 1 | 1 |
| 1 | 0 | 1 | 1 | 1 |
| 1 | 1 | 0 | 1 | 1 |
| 1 | 1 | 1 | 1 | 1 |

The results observed experimentally for one chip circuit from MAX 7000 FGPA were identical to results obtained theoretically as expected.

Multiple Output Circuit

- Table 14: Comparison of Theoretical and Experimental results for Multiple Output logic circuit

| Inputs |  |  | Expected Results |  | Actual Results |  |
| --- | --- | --- | --- | --- | --- | --- |
| A3 | B3 | D3 | S3 | S3 | C3 | C3 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 | 0 | 0 |
| 0 | 1 | 0 | 1 | 1 | 0 | 0 |
| 0 | 1 | 1 | 0 | 0 | 1 | 1 |
| 1 | 0 | 0 | 1 | 1 | 0 | 0 |
| 1 | 0 | 1 | 0 | 0 | 1 | 1 |
| 1 | 1 | 0 | 0 | 0 | 1 | 1 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 |

The results observed experimentally for one chip circuit from MAX 7000 FGPA were identical to results obtained theoretically as expected.